System Architecture Document

Project Name:  Xin1 Integrated Powertrain Domain Controller

Technology Owner:

Document Version: V001

Document No:  Xin1_SAS_001

Change History

| Version | Date | Author | Reviewer | Approver | Change description |
| --- | --- | --- | --- | --- | --- |
| 001 | 20/07/2026 | Ratneswar Sahoo | Pankaj D | Rajiv Kumar | Initial Draft |

TABLE OF CONTENTS

1	Introduction	4

1.1	Document scope	4

1.2	Terminologies	4

2	System Requirements	5

2.1	System overview	5

3	System Architecture	6

3.1	Introduction	6

3.2	Block diagram/ boundary diagram	7

3.3	List of system architecture elements:	7

3.4	System Relationship	8

3.5	Dynamic behavior of the system and its elements:	13

3.6	Operation mode and its dependencies	13

4	Generic Requirements:	13

4.1	Tata Motors Generic Requirements	13

4.2	Relevant Industry Standards	13

5	System Integration plan:	14

5.1	System Integration approach	14

6	System Architecture Signoff details	14

7	Appendix:	15

# Introduction

## Document scope

This document describes the system architecture of the Xin1 Integrated Powertrain Unit.

The architecture defines:

- System boundary

- Architectural elements

- Internal and external interfaces

- Functional decomposition

- Allocation of system functions

- Interaction among architectural elements

- Operating modes

- Integration strategy

The architecture serves as the baseline for:

- System Design (SYS.3)

- Hardware Design

- Software Design

- Cybersecurity Concept Allocation

- Verification & Validation Activities

## Terminologies

| Acronym | Definition |
| --- | --- |
| CAN | Controller Area Network |
| CAN FD | Controller Area Network Flexible Data-rate |
| CCS2 | Combined Charging System Type 2 |
| DTC | Diagnostic Trouble Code |
| EOL | End-of-Line |
| EVCU | Electric Vehicle Control Unit |
| OTA | Over-the-Air |
| PLC | Power Line Communication |
| UDS | Unified Diagnostic Services |
| MCU | Motor Control Unit |
| HV | High Voltage |
| LV | Low Voltage |
| PDU | Power Distribution Unit |
| HVIL | High Voltage Interlock Loop |
| HSD | High Side Driver |
| LSD | Low Side Driver |
| ASIL | Automotive Safety Integrity Level |
| FuSa | Functional Safety |
| EVSE | Electric Vehicle Supply Equipment |
| HIL | Hardware-In-Loop |

Terminologies

# System Requirements

## System overview

The Xin1 system is an integrated electric powertrain domain control platform that combines multiple vehicle electrification functions into a single domain architecture operating on a 650 V/800 V high-voltage battery system.

### System Description

The system includes:

- 250 kW traction inverter (main propulsion)

- 6 kW DC/DC converter (HV to LV conversion)

- 10 kW steering auxiliary inverter

- 10 kW brake pump auxiliary inverter

- Electric Vehicle Control Unit (EVCU)

- CCS2 charging control module

- Vehicle communication gateway (CAN/Ethernet)

The system receives HV power from the battery pack via the PDU and distributes electrical energy to traction, steering, braking and LV power networks.

### Reference document

#### System Requirement document

Please refer to “System Requirement Document_V001”.

# System Architecture

## Introduction

The Xin1 System is divided into following element groups:

Control Domain

- Main CPU (Contains eVCU)

- Sub CPU

Power Domain

- Traction Inverter-1 Power board

- Traction Inverter-2 Power board

- Steering Auxiliary Inverter Power board

- Brake Auxiliary Inverter Power board

- DCDC Converter-1 Power board

- DCDC Converter-2 Power board

- DCDC Converter-3 Power board

## Block diagram/ boundary diagram

## List of system architecture elements:

| Element ID | Element | Sub-element |
| --- | --- | --- |
| SYS_ARCH_ELE_ID_01 | Control Domain | Main CPU (Contains eVCU) |
| SYS_ARCH_ELE_ID_02 | Control Domain | Sub CPU |
| SYS_ARCH_ELE_ID_03 | Power Domain | Traction Inverter-1 power board |
| SYS_ARCH_ELE_ID_04 | Power Domain | Traction Inverter-2 power board |
| SYS_ARCH_ELE_ID_05 | Power Domain | Steering Auxiliary Inverter power board |
| SYS_ARCH_ELE_ID_06 | Power Domain | Brake Auxiliary Inverter power board |
| SYS_ARCH_ELE_ID_07 | Power Domain | DCDC Converter-1 (6kW) power board |
| SYS_ARCH_ELE_ID_08 | Power Domain | DCDC Converter-2 (6kW) power board |
| SYS_ARCH_ELE_ID_09 | Power Domain | DCDC Converter-3 (1kW) power board |
| SYS_ARCH_ELE_ID_10 | External Interface | HVIL |
| SYS_ARCH_ELE_ID_11 | External Interface | CCS2 charging |
| SYS_ARCH_ELE_ID_12 | External Interface | Analog inputs |
| SYS_ARCH_ELE_ID_13 | External Interface | Digital inputs/outputs |
| SYS_ARCH_ELE_ID_14 | External Interface | HSD/LSD |
| SYS_ARCH_ELE_ID_15 | External Interface | Battery Pack via PDU |
| SYS_ARCH_ELE_ID_16 | External Interface | Cooling System |

List of System Architecture elements

## System Relationship

### Relationship of system elements (including interfaces and interactions)

| Element ID | Element | Sub-element | Category (SW/HW/Both) | Function | Related with Element/SW | Type of interface (Internal/External) | Interface Interaction description | Detailed Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SYS_ARCH_ELE_01 | Control Domain | Main CPU (Contains EVCU) | Both | Vehicle control, state management, diagnostics, safety management | Sub CPU | Internal | CAN Communication | EVCU exchanges commands, diagnostics, watchdog status, and safety monitoring information with Sub CPU. |
|  |  |  |  | Torque coordination | Traction Inverter-1 | Internal | CAN Communication | EVCU transmits propulsion enable, torque request, derating command and receives motor feedback and inverter diagnostics. |
|  |  |  |  | Torque coordination | Traction Inverter-2 | Internal | CAN Communication | EVCU transmits propulsion commands and receives inverter status and fault information. |
|  |  |  |  | Steering control coordination | Steering Auxiliary Inverter | Internal | CAN Communication | EVCU sends steering enable and operating mode commands and receives diagnostics/status. |
|  |  |  |  | Brake system coordination | Brake Auxiliary Inverter | Internal | CAN Communication | EVCU sends compressor requests and receives motor status, faults and operational feedback. |
|  |  |  |  | LV power management | DCDC Converter-1 | Internal | CAN Communication | EVCU supervises converter operation and receives voltage/current/temperature diagnostics. |
|  |  |  |  | LV power management | DCDC Converter-2 | Internal | CAN Communication | EVCU supervises converter operation and receives status information. |
|  |  |  |  | Auxiliary battery charging | DCDC Converter-3 | Internal | CAN Communication | EVCU supervises 12V charging operation and diagnostic information. |
| SYS_ARCH_ELE_02 | Control Domain | Sub CPU | Both | Safety supervision | EVCU Main CPU | Internal | CAN Communication | Sub CPU performs supervision, diagnostics and watchdog monitoring of VECU. |
| SYS_ARCH_ELE_03 | Power Domain | Traction Inverter-1 power board | Both | Motor drive control | Traction Motor-1 | External | 3-Phase Power Interface (RYB) | Converts HV DC bus voltage into controlled 3-phase AC power for propulsion motor. |
| SYS_ARCH_ELE_04 | Power Domain | Traction Inverter-2 power board | Both | Motor drive control | Traction Motor-2 | External | 3-Phase Power Interface (RYB) | Converts HV DC bus voltage into controlled 3-phase AC power for propulsion motor. |
| SYS_ARCH_ELE_05 | Power Domain | Steering Auxiliary Inverter power board | Both | Steering pump motor control | Steering Pump Motor | External | 3-Phase Power Interface (RYB) | Controls steering motor speed and torque according to vehicle command. |
| SYS_ARCH_ELE_06 | Power Domain | Brake Auxiliary Inverter power board | Both | Brake compressor motor control | Brake Compressor Motor | External | 3-Phase Power Interface (RYB) | Controls brake compressor motor operation and pressure generation. |
| SYS_ARCH_ELE_07 | Power Domain | DCDC Converter-1 (6kW) power board | Both | LV power supply, LV battery charging | Vehicle 24V Loads | External | 24V Power Output | Supplies regulated 24V power to vehicle ECUs, loads and charges 24V LV battery. |
| SYS_ARCH_ELE_08 | Power Domain | DCDC Converter-2 (6kW) power board | Both | LV power supply | Vehicle 24V Loads | External | 24V Power Output | Provides additional 24V supply capacity and redundancy. |
| SYS_ARCH_ELE_09 | Power Domain | DCDC Converter-3 (1kW) power board | Both | Auxiliary battery charging | 12V loads | External | 12V Power Output | Charges auxiliary battery and supplies low power loads. |
| SYS_ARCH_ELE_10 | Vehicle Interface | HVIL | HW | High voltage safety monitoring | EVCU Main CPU | External | Digital Safety Signal | Indicates continuity of HV interlock loop. Loss of HVIL triggers safety action. |
| SYS_ARCH_ELE_11 | Vehicle Interface | CCS2 charging | Both | Charging communication | EVCU Main CPU | External | PLC Communication | Supports ISO 15118 and CCS2 charging communication with EVSE. |
| SYS_ARCH_ELE_12 | Vehicle Interface | Analog Inputs | HW | Vehicle sensing | EVCU Main CPU | External | Analog Interface | Inputs such as pedal sensors, pressure sensors, temperature sensors and resolver. |
| SYS_ARCH_ELE_13 | Vehicle Interface | Digital Inputs/Outputs | HW | Vehicle control | EVCU Main CPU | External | Digital IO | Wake-up, ignition, interlock and control signals. |
| SYS_ARCH_ELE_14 | Vehicle Interface | HSD/LSD | HW | Actuator control | EVCU Main CPU | External | High Side/Low Side Driver Interface | Used for relay, actuator and auxiliary load control. |
| SYS_ARCH_ELE_15 | External Power Source | Battery Pack via PDU | HW | HV power source | Internal HV Distribution Bus | External | HV Power Interface | Provides HV DC power to all Xin1 power conversion and propulsion aggregates. |
| SYS_ARCH_ELE_16 | Thermal Management | Cooling System | HW | Heat removal | All Power Domain Elements | External | Coolant Flow Interface | Removes heat from traction inverters, auxiliary inverters and DCDC converters through coolant circulation. |

### Allocation matrix

## Dynamic behavior of the system and its elements:

## Operation mode and its dependencies

| Operation mode | State | Command | Time (msec) | Description |
| --- | --- | --- | --- | --- |
| Initialization | Init | Ignition ON | <500 | System startup and self-test |
| Idle | Standby | Wake-up | Continuous | Waiting for drive request |
| Drive | Ready | Drive enable | <100 | Torque generation enabled |
| Charging | Charge | EVSE connect | Continuous | Charging operation active |
| Derating | Derate | Fault active | <50 | Reduced performance mode |
| Fault | Safe state | Critical fault | <100 | Safe state active |
| Off | Shutdown | Ignition off | <1000 | Controlled shutdown |

Operation modes

# Generic Requirements:

## Tata Motors Generic Requirements

| Document Number | Definition |
| --- | --- |

List of Tata motors generic requirements

## Relevant Industry Standards

| Document Number | Definition |
| --- | --- |
| ISO 26262 | Functional Safety |
| ASPICE v4.0 | Automotive Process Assessment |
| ISO 21434 | Automotive Cybersecurity |
| ISO 11898 | CAN Communication |
| ISO 14229 | UDS Diagnostics |
| ISO 15118 | EV-EVSE Communication |

List of Relevant Industry Standards

# System Integration plan:

## System Integration approach

| Step | Description |
| --- | --- |
| 1 | Main CPU integration |
| 2 | Sub CPU integration |
| 3 | Traction Inverter-1 integration |
| 4 | Traction Inverter-2 integration |
| 5 | Steering Inverter integration |
| 6 | Brake Inverter integration |
| 7 | DCDC Converter integration |
| 8 | CCS2 Charging integration |
| 9 | Gateway/Ethernet integration |
| 10 | HVIL integration |
| 11 | System HIL integration |
| 12 | Lab-Car integration |
| 13 | Vehicle integration |
| 14 | Vehicle validation |
| 15 | Release sign-off |

### System Integration steps and their sequence

| Step | Description |
| --- | --- |

List of System Integration and their sequence

# System Architecture Signoff details

| Sr.no | Function | Name | Designation |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |

List of stakeholder for signoff

# Appendix: