System Requirements Document

Project Name: Xin1 Integrated Powertrain Domain Controller

Technology Owner:

Document Version: V001

Document Name: Xin1_SyRS_001

Change History

| Version | Date | Author | Reviewer | Approver | Change description |
| --- | --- | --- | --- | --- | --- |
| 001 | 10/07/2026 | Ratneswar Sahoo | Pankaj D | Rajiv Kumar | Initial Draft |

TABLE OF CONTENTS

1	Introduction	4

1.1	Document Scope	4

1.2	Requirements Definition	4

1.3	Terminologies	5

2	Project Requirements	5

2.1	Purpose	5

2.2	Feature List	6

3	System Description	7

3.1	Introduction	7

3.2	System Overview / Block Diagram	8

3.3	ECU Interface	8

4	Embedded System Requirements:	8

4.1	Design & External Interface Requirements	9

4.2	Diagnostics Requirements	9

4.3	Functional Safety (FuSa) and Cyber Security Requirements	11

4.4	Testing and Validation Requirements	13

5	User Interface Requirement	14

6	Considerations/Constraints (Sys / HW)	14

7	Other Requirements	14

7.1	Project Timeline	14

7.2	Maintenance Schedule	14

7.3	Calibration & Code Flashing Sequence	14

7.4	Warranty	14

7.5	Logistical Requirements:	15

8	System requirements Signoff details	15

9	Appendix	15

# Introduction

## Document Scope

This document defines the complete set of system-level requirements for Xin1 integrated electric powertrain domain control system.

The requirement describe what the integrated system shall accomplish and do not prescribe implementation details unless required by external standards or vehicle-level constraints.

The document includes requirements for:

• Vehicle operating modes

• High voltage power management

• Traction Inverter

• DC/DC converter

• Steering auxiliary inverter

• Brake pump auxiliary inverter

• Vehicle control

• Charging control

• Vehicle communication

• Diagnostics

• Protection

• Cybersecurity

• Environmental performance

## Requirements Definition

The interpretation of different requirements is controlled by the following definitions:

- Shall, Must - A binding requirement (verifiable) of the component within the scope of this specification.

- Should – A non-binding requirement (recommendation) of the component within the scope of this specification.

- Will - A requirement that is implemented outside of the scope of this specification, that is required for the successful implementation of this specification.

- May – This word in the text expresses a permissible practice or action. It does not express a requirement of the specification.

## Terminologies

| Acronym | Definition |
| --- | --- |
| AGG | Aggregate |
| ASIL | Automotive Safety Integrity Level |
| AUTOSAR | AUTomotive Open System ARchitecture |
| BMS | Battery Management System |
| CAN | Controller Area Network |
| CAN FD | Controller Area Network Flexible Data-rate |
| CCS2 | Combined Charging System Type 2 |
| DTC | Diagnostic Trouble Code |
| EOL | End-of-Line |
| EVCU | Electric Vehicle Control Unit |
| FSC | Functional Safety Concept |
| HARA | Hazard Analysis and Risk Assessment |
| OTA | Over-the-Air |
| PLC | Power Line Communication |
| PSFB | Phase-Shift Full-Bridge |
| SyRS | System Requirements Specification |
| TARA | Threat Analysis and Risk Assessment |
| TSR | Technical Safety Requirement |
| UDS | Unified Diagnostic Service |

Table 1: Terminologies

# Project Requirements

## Purpose

The Xin1 Integrated Powertrain Unit shall integrate propulsion control, auxiliary motor drive control, low-voltage power generation and vehicle control functions within a single enclosure.

The Xin1 system shall:

- Convert HV battery energy into traction power.

- Supply regulated LV power to vehicle loads.

- Control steering pump motor operation.

- Control brake compressor motor operation.

- Coordinate propulsion and regenerative braking.

- Support EV charging communication.

- Ensure functional safety and diagnostic functionality.

- Support commercial vehicle applications including HCV, ILM and Bus variants.

## Feature List

### Stakeholders Description

The following table contains details of all project stakeholders.

| Sr.no | Function | Name | Designation |
| --- | --- | --- | --- |
| 1 | System Engineering |  | System Architect |
| 2 | Functional Safety |  | FuSa Manager |
| 3 | Power Electronics |  | Technical Lead |
| 4 | Software Development |  | SW Architect |
| 5 | Hardware Development |  | HW Architect |
| 6 | Validation |  | Test Manager |
| 7 | Program Management |  | Project Manager |
| 8 | Integration |  |  |

Stakeholder details

### List of features / Broad vehicle level requirements

| Veh_REQ_ID | Vehicle Level Requirement / PAT requirement | Remarks |
| --- | --- | --- |
| Veh_REQ_01 |  |  |

The given table below contains the list of required features / vehicle level requirements / Applicable PAT requirements.

List of vehicle level requirements

### Applicable standards/regulations

| Document Number | Definition |
| --- | --- |
| ISO 26262 | Road Vehicle Functional Safety |
| ISO 21434 | Automotive Cybersecurity |
| ISO 15118 | EV Charging Communication |
| ASPICE v4.0 | Process Assessment Model |
| UNECE R155 | Cybersecurity Regulation |
| UNECE R156 | Software Update Regulation |
| AIS-038 | Electric Vehicle Requirements |
| SAE J1939 | Vehicle Communication |

# System Description

## Introduction

The Xin1 system is an integrated electric powertrain domain control platform that combines multiple vehicle electrification functions into a single domain architecture operating on a 650 V high-voltage battery system.

The system includes:

- 250 kW traction inverter (main propulsion) (2x)

- 6 kW DC/DC converter (HV to LV conversion) (2x)

- 1 kW DC/DC converter (HV to LV conversion) (1x)

- 10 kW steering auxiliary inverter (1x)

- 10 kW brake pump auxiliary inverter (braking system support) (1x)

- Electric Vehicle Control Unit (EVCU)

- CCS2 charging control

- Vehicle communication gateway (CAN/Ethernet)

## System Overview / Block Diagram

## ECU Interface

| Interface ID | Interface Name | Direction | Description |
| --- | --- | --- | --- |
| IF_01 | HV Input | Input | Battery pack HV supply through PDU |
| IF_02 | HVIL | Input | HV safety monitoring |
| IF_03 | Analog Inputs | Input | Sensor acquisition |
| IF_04 | Digital Inputs | Input | Vehicle discrete signals |
| IF_05 | HSD/LSD | I/O | Actuator controls |
| IF_06 | PLC | Bidirectional | EVSE communication |
| IF_07 | CAN | Bidirectional | Internal aggregate communication |
| IF_08 | Cooling IN | Input | Coolant inlet |
| IF_09 | Cooling OUT | Output | Coolant outlet |
| IF_10 | RYB Output | Output | Motor drive outputs |
| IF_11 | 24V Output | Output | LV load supply |
| IF_12 | 12V Output | Output | Auxiliary battery charging |

# Embedded System Requirements:

## Design & External Interface Requirements

| Req_ID | Req_Name | Req_Description | Priorities (H/M/L) | Categories (Critical/Non-critical) |
| --- | --- | --- | --- | --- |
| General Requirements |  |  |  |  |
|  | Operating Voltage Range | The system shall operate normally when supplied with an input voltage within the range specified in the system architecture document. | H | Critical |
|  | System Power-Up | The system shall transition to Initialization state within “T” s after application of vehicle supply power. | H | Critical |
|  | Startup Time | The system shall complete initialization and enter operational mode within “T” s after power-up. | H | Critical |
|  | System Self-Test | The system shall execute a startup diagnostic check of all safety-related functions during initialization. | H | Critical |
|  | Operational Status | The system shall provide current system operating status to connected vehicle systems. | H | Critical |
|  | Fault Logging | The system shall store all detected faults in non-volatile memory within T sec of Flt Detection | H | Critical |
|  | Time Stamp Storage | The system shall associate each stored fault event with a timestamp. | H | Critical |
|  | Safe-State Entry | The system shall enter a safe state upon detection of a critical fault condition. | H | Critical |
|  | Controlled Shutdown | The system shall perform a controlled shutdown when a shutdown request is received. | H | Critical |
|  | Voltage Monitoring | The system shall continuously monitor input supply voltage during operation. | H | Critical |
|  | Temperature Monitoring | The system shall continuously monitor system temperature during operation. | H | Critical |
|  | Diagnostic Reporting | The system shall report detected diagnostic events via diagnostic communication interface. | H | Critical |
|  | Environmental Operating Range | The system shall operate within the ambient temperature range defined in the product specification. | H | Critical |
| Communication Protocol Requirements |  |  |  |  |
|  | CAN Support | The system shall support CAN communication compliant with ISO 11898. | H | Critical |
|  | UDS Support | The system shall support diagnostic services compliant with ISO 14229. | H | Critical |
|  | ISO15118 Support | The system shall support EV-EVSE communication compliant with ISO 15118. | H | Critical |
|  | PLC Communication | The system shall support Power Line Communication between vehicle and EVSE. | H | Critical |
|  | Message Monitoring | The system shall monitor reception of mandatory communication messages. | H | Critical |
|  | Communication Timeout | The system shall detect communication timeout when a mandatory message is not received within the configured timeout period. | H | Critical |
|  | Communication Recovery | The system shall automatically resume communication following recovery from a temporary communication interruption. | M | Non-Critical |
|  | Data Integrity Check | The system shall validate communication message integrity using protocol- specific integrity checks before processing received data. | H | Critical |
|  | Session Establishment | The system shall establish communication sessions before exchanging charging control information. | H | Critical |
|  | Session Termination | The system shall terminate communication sessions when charging termination conditions are met. | H | Critical |
| Functional Requirements |  |  |  |  |
|  |  | Vehicle power management requirements |  |  |
|  | Vehicle Power Mode Management | The system shall manage transitions between Off, Wake-up, Ready, Drive, Charging and Shutdown vehicle power modes. | H | Critical |
|  | Power Mode Coordination | The system shall coordinate operating modes of all Xin1 aggregates according to vehicle operating state. | H | Critical |
|  | Aggregate Activation | The system shall activate only the aggregates required for the current vehicle operating mode. | H | Critical |
|  | Aggregate Deactivation | The system shall deactivate non-required aggregates during vehicle shutdown. | H | Critical |
|  | HV Availability Monitoring | The system shall determine availability of HV power before enabling HV-dependent functions. | H | Critical |
|  | Energy Flow Supervision | The system shall supervise energy flow between HV source and connected aggregates. | H | Critical |
|  | Fault-Tolerant Operation | The system shall maintain operation of unaffected aggregates following detection of a non-critical aggregate fault. | H | Critical |
|  | Vehicle Safe State Control | The system shall initiate vehicle safe-state actions following detection of a critical system fault. | H | Critical |
|  |  | EVCU Functional Requirements |  |  |
|  | Vehicle State Determination | The system shall determine overall vehicle operational state using valid vehicle inputs. | H | Critical |
|  | System State Management | The system shall manage operation using predefined system states and state transitions. | H | Critical |
|  | Torque Coordination | The system shall provide propulsion torque requests to the traction inverter. | H | Critical |
|  | Aggregate Command Management | The system shall manage operational commands issued to Xin1 aggregates. | H | Critical |
|  | Vehicle Interlock Monitoring | The system shall monitor vehicle interlock conditions before enabling torque production. | H | Critical |
|  | Drive Enable Management | The system shall enable vehicle propulsion only when all propulsion preconditions are satisfied. | H | Critical |
|  | Operational Monitoring | The system shall continuously monitor all safety-related inputs during operation. | H | Critical |
|  | Status Reporting | The system shall provide current operating status information to connected systems at the configured transmission rate. | H | Critical |
|  | Shutdown Control | The system shall coordinate orderly shutdown of Xin1 aggregates during vehicle power-down. | H | Critical |
|  | Fault Handling | The system shall transition to a safe state upon detection of a critical fault. | H | Critical |
|  | Vehicle Diagnostics Coordination | The system shall collect system-level health information from Xin1 aggregates. | H | Critical |
|  |  | Traction Inverter Functional Requirements |  |  |
|  | Propulsion Request Processing | The system shall provide propulsion commands to the traction inverter when propulsion is permitted. | H | Critical |
|  | Regenerative Braking Coordination | The system shall support regenerative braking requests during vehicle deceleration. | H | Critical |
|  | Torque Inhibit Management | The system shall inhibit traction torque upon receipt of a valid torque inhibit request. | H | Critical |
|  | Traction Availability Monitoring | The system shall monitor traction inverter operational availability. | H | Critical |
|  | Derating Handling | The system shall manage propulsion operation according to traction inverter derating information. | H | Critical |
|  | Propulsion Fault Response | The system shall remove propulsion request following detection of a critical traction inverter fault. | H | Critical |
|  | Direction Control | The system shall support propulsion operation according to selected vehicle travel direction. | H | Critical |
|  | Torque Feedback Monitoring | The system shall monitor torque feedback information provided by the traction inverter. | H | Critical |
|  | Dual Inverter Coordination | The system shall coordinate operation of both traction inverters during propulsion. | H | Critical |
|  | Torque Demand Distribution | The system shall distribute requested vehicle torque among available traction inverters. | H | Critical |
|  | Torque Balance Monitoring | The system shall monitor delivered torque contribution from both traction inverters. | H | Critical |
|  | Single Inverter Fault Handling | The system shall support degraded propulsion operation upon loss of one traction inverter when vehicle safety permits. | H | Critical |
|  | Regenerative Braking Coordination | The system shall coordinate regenerative braking torque among available traction inverters. | H | Critical |
|  |  | 6 kW DCDC Converter Functions |  |  |
|  | LV Supply Support | The system shall provide low-voltage system support through the 6 kW DCDC converter when HV power is available. | H | Critical |
|  | Converter Enable Control | The system shall control activation of the 6 kW DCDC converter according to vehicle operating mode. | H | Critical |
|  | Converter Health Monitoring | The system shall monitor operational status of the 6 kW DCDC converter. | H | Critical |
|  | Output Availability Monitoring | The system shall monitor availability of low-voltage output from the 6 kW DCDC converter. | H | Critical |
|  | Fault Handling | The system shall report faults detected in the 6 kW DCDC converter. | H | Critical |
|  | Dual DCDC Load Sharing | The system shall coordinate operation of the two 6 kW DCDC converters when both are available. | H | Critical |
|  | DCDC Redundancy Management | The system shall support continued LV power generation following failure of one 6 kW DCDC converter when system capability permits. | H | Critical |
|  |  | 1 kW DCDC Converter Functions |  |  |
|  | Auxiliary LV Support | The system shall support low-voltage auxiliary loads through the 1 kW DCDC converter. | M | Non-Critical |
|  | Operational Monitoring | The system shall monitor operational status of the 1 kW DCDC converter. | M | Non-Critical |
|  | Converter Fault Detection | The system shall detect faults affecting operation of the 1 kW DCDC converter. | M | Non-Critical |
|  | Converter Availability Reporting | The system shall report availability status of the 1 kW DCDC converter. | M | Non-Critical |
|  |  | Steering Auxiliary Inverter Functional Requirements |  |  |
|  | Steering Power Supply | The system shall provide electrical power for steering assistance operation. | H | Critical |
|  | Steering Enable Control | The system shall enable steering assistance only when steering operation conditions are satisfied. | H | Critical |
|  | Steering Availability Monitoring | The system shall monitor steering inverter operational status. | H | Critical |
|  | Steering Fault Detection | The system shall detect steering aggregate faults. | H | Critical |
|  | Steering Degraded Mode | The system shall report steering degraded operation when steering functionality is limited. | H | Critical |
|  | Steering Safe-State Support | The system shall support execution of steering system safe-state actions following a critical steering fault. | H | Critical |
|  | Steering Shutdown Control | The system shall execute controlled steering aggregate shutdown during vehicle power-down. | H | Critical |
|  |  | Brake Pump Auxiliary Inverter Functional Requirements |  |  |
|  | Brake Power Support | The system shall provide electrical power to the brake pump aggregate. | H | Critical |
|  | Brake Pump Enable Control | The system shall enable brake pump operation when requested by vehicle braking functions. | H | Critical |
|  | Brake Aggregate Monitoring | The system shall monitor brake pump operational status. | H | Critical |
|  | Brake Aggregate Fault Detection | The system shall detect brake aggregate faults. | H | Critical |
|  | Brake Function Availability Reporting | The system shall report brake aggregate availability status. | H | Critical |
|  | Brake Safe-State Management | The system shall initiate brake safe-state actions when a critical brake aggregate fault is detected. | H | Critical |
|  |  | Charging System Functions (CCS2) |  |  |
|  | Charging Connection Detection | The system shall detect connection of a CCS2 charging connector. | H | Critical |
|  | Charging Session Management | The system shall manage charging sessions according to CCS2 charging procedures. | H | Critical |
|  | Charging Authorization Management | The system shall initiate charging only after successful charging authorization. | H | Critical |
|  | EVSE Communication Management | The system shall exchange charging information with EVSE during charging operation. | H | Critical |
|  | Charging Current Limitation | The system shall ensure charging operation remains within communicated charging limits. | H | Critical |
|  | Charging Fault Response | The system shall terminate charging upon detection of critical charging faults. | H | Critical |
|  | Charging Completion Handling | The system shall terminate charging when charging completion conditions are satisfied. | H | Critical |
|  | Charging Status Reporting | The system shall provide charging status information to vehicle systems. | M | Non-Critical |
|  |  | Communication Gateway Functions |  |  |
|  | CAN Communication Routing | The system shall route authorized CAN messages between connected vehicle networks. | H | Critical |
|  | Ethernet Communication Routing | The system shall route authorized Ethernet communication between connected systems. | H | Critical |
|  | Data Exchange Management | The system shall support exchange of operational information among Xin1 aggregates. | H | Critical |
|  | Communication Fault Detection | The system shall detect communication failures affecting vehicle operation. | H | Critical |
|  | Gateway Health Monitoring | The system shall monitor gateway operational status. | H | Critical |
|  | Synchronization Support | The system shall support synchronization of vehicle-level information exchanged between aggregates. | M | Non-Critical |
|  |  | System Diagnostics and Safety Functions |  |  |
|  | Aggregate Fault Monitoring | The system shall monitor fault status of all Xin1 aggregates. | H | Critical |
|  | Diagnostic Trouble Code Storage | The system shall store detected system-level faults as diagnostic trouble codes. | H | Critical |
|  | Fault Propagation Management | The system shall communicate critical fault conditions to affected vehicle systems. | H | Critical |
|  | HVIL Monitoring | The system shall monitor HVIL status during vehicle operation and charging. | H | Critical |
|  | Crash Event Response | The system shall initiate system shutdown actions following receipt of a crash event indication. | H | Critical |
|  | Safe Torque Off Coordination | The system shall inhibit propulsion torque following detection of a safety-critical event. | H | Critical |
|  | System Health Reporting | The system shall provide Xin1 system health status to vehicle diagnostic systems. | H | Critical |
|  |  | HV Power Distribution & PDU Functions |  |  |
|  | HV Power Distribution Management | The system shall control distribution of HV power to connected aggregates based on vehicle operating state. | H | Critical |
|  | HV Bus Energization | The system shall energize the HV bus only after successful completion of all HV enabling conditions. | H | Critical |
|  | HV Bus De-energization | The system shall de-energize the HV bus upon vehicle shutdown or critical safety event detection. | H | Critical |
|  | Pre-charge Request Management | The system shall initiate HV pre-charge before enabling HV power delivery to connected loads. | H | Critical |
|  | Pre-charge Completion Verification | The system shall verify successful HV pre-charge completion before closing main contactors. | H | Critical |
|  | Contactors Status Monitoring | The system shall monitor status feedback from HV contactors. | H | Critical |
|  | Welded Contactor Detection | The system shall detect abnormal contactor behavior indicative of a welded contactor condition. | H | Critical |
|  | HV Distribution Fault Handling | The system shall remove HV power from affected aggregates upon detection of a critical HV distribution fault. | H | Critical |
|  |  | HV Battery Integration Functions |  |  |
|  | HV Battery Status Acquisition | The system shall receive battery status information from the BMS. | H | Critical |
|  | SOC Monitoring | The system shall monitor battery State of Charge for system operation management. | H | Critical |
|  | SOH Monitoring | The system shall monitor battery State of Health information when available from the BMS. | M | Non-Critical |
|  | Battery Power Limit Compliance | The system shall ensure aggregate power demand remains within battery power limits communicated by the BMS. | H | Critical |
|  | Charge Power Limit Compliance | The system shall ensure charging power remains within BMS-authorized charging limits. | H | Critical |
|  | Discharge Power Limit Compliance | The system shall ensure propulsion power remains within BMS-authorized discharge limits. | H | Critical |
|  | Battery Fault Handling | The system shall initiate protective actions upon receipt of a battery critical fault indication. | H | Critical |
|  | HV Battery Status Acquisition | The system shall receive battery status information from the BMS. | H | Critical |
|  |  | Main CPU and Sub-CPU Coordination Functions |  |  |
|  | Controller Synchronization | The system shall maintain synchronization between Main CPU and Sub-CPU functions. | H | Critical |
|  | Internal Communication Monitoring | The system shall monitor communication integrity between Main CPU and Sub-CPU. | H | Critical |
|  | Controller Fault Detection | The system shall detect loss of communication with either processing unit. | H | Critical |
|  | Controller Degraded Operation | The system shall enter a defined degraded mode following loss of non-critical processing functionality. | H | Critical |
|  |  | Thermal System Functions |  |  |
|  | Coolant Status Monitoring | The system shall monitor availability of coolant flow information when provided by external systems. | H | Critical |
|  | Aggregate Temperature Supervision | The system shall supervise thermal status of all power electronic aggregates. | H | Critical |
|  | Thermal Derating Coordination | The system shall coordinate aggregate derating based on thermal limits. | H | Critical |
|  | Thermal Fault Handling | The system shall initiate protective actions when aggregate thermal limits are exceeded. | H | Critical |
|  |  | Energy and Power Management Functions |  |  |
|  | Aggregate Power Demand Monitoring | The system shall monitor power demand of connected Xin1 aggregates. | H | Critical |
|  | Power Budget Management | The system shall manage aggregate operation within available vehicle power limits. | H | Critical |
|  | Load Prioritization | The system shall prioritize safety-related loads during power limitation conditions. | H | Critical |
|  | Controlled Load Shedding | The system shall shed non-critical loads when available power becomes insufficient. | H | Critical |
|  |  | Functional Degradation and Limp-Home Requirements |  |  |
|  | Degraded Mode Determination | The system shall determine vehicle operating capability based on the health status of all Xin1 aggregates. | H | Critical |
|  | Limp-Home Activation | The system shall support a limp-home operating mode when propulsion capability is partially available. | H | Critical |
|  | Propulsion Derating | The system shall limit available propulsion power when a degraded operating condition is active. | H | Critical |
|  | Auxiliary Load Prioritization | The system shall prioritize steering and braking functions during degraded operation. | H | Critical |
|  | Driver Notification of Degradation | The system shall provide indication of active degraded operating modes to the vehicle HMI. | H | Critical |
|  | Degradation Recovery | The system shall exit degraded mode only after recovery conditions are satisfied. | H | Critical |
|  |  | HVIL and Safety Interlock Requirements |  |  |
|  | HVIL Monitoring | The system shall continuously monitor the HVIL status during vehicle operation and charging. | H | Critical |
|  | HVIL State Validation | The system shall verify HVIL integrity before HV bus energization. | H | Critical |
|  | HVIL Open Response | The system shall initiate protective actions upon detection of an HVIL open condition. | H | Critical |
|  | HVIL Fault Reporting | The system shall report HVIL faults to vehicle diagnostic systems. | H | Critical |
|  | HV Enable Prevention | The system shall prevent HV activation when HVIL integrity is not confirmed. | H | Critical |
|  |  | Crash and Emergency Event Requirements |  |  |
|  | Crash Signal Monitoring | The system shall monitor crash event indications received from vehicle safety systems. | H | Critical |
|  | Crash HV Shutdown | The system shall de-energize the HV bus upon receipt of a valid crash indication. | H | Critical |
|  | Torque Inhibition after Crash | The system shall inhibit propulsion torque following a crash indication. | H | Critical |
|  | Charging Termination after Crash | The system shall terminate charging following a crash indication. | H | Critical |
|  | Crash Event Logging | The system shall store crash-related operational information for diagnostics. | H | Critical |
|  |  | OTA/FOTA Requirements |  |  |
|  | Software Update Support | The system shall support authorized software update functionality. | H | Critical |
|  | Software Package Validation | The system shall verify integrity and authenticity of update packages before installation. | H | Critical |
|  | Update Status Reporting | The system shall report software update status information. | M | Non-Critical |
|  | Failed Update Handling | The system shall prevent activation of invalid software following unsuccessful update completion. | H | Critical |
|  | Software Version Reporting | The system shall provide installed software version information through diagnostic interfaces. | M | Non-Critical |
|  |  | Vehicle Wake-up and Sleep Management Requirements |  |  |
|  | Wake-up Detection | The system shall detect valid vehicle wake-up requests. | H | Critical |
|  | Aggregate Wake-up Coordination | The system shall coordinate startup of Xin1 aggregates following wake-up. | H | Critical |
|  | Wake-up Sequence Control | The system shall execute aggregate startup according to the configured startup sequence. | H | Critical |
|  | Sleep Entry Determination | The system shall determine eligibility for vehicle sleep mode. | H | Critical |
|  | Sleep Mode Coordination | The system shall coordinate transition of aggregates into sleep mode. | H | Critical |
|  | Sleep Prevention | The system shall prevent entry into sleep mode while critical operations are active. | H | Critical |
|  | Wake-up Failure Detection | The system shall detect unsuccessful aggregate wake-up attempts. | H | Critical |
|  | Low Power Monitoring | The system shall monitor aggregate status during low power mode. | M | Non-Critical |
|  |  | Communication Management Requirements |  |  |
|  | CAN Network Management | The system shall participate in vehicle CAN network management. | H | Critical |
|  | Ethernet Network Management | The system shall participate in vehicle Ethernet network management. | H | Critical |
|  | Communication Availability Monitoring | The system shall monitor availability of vehicle communication networks. | H | Critical |
|  | Bus-Off Detection | The system shall detect CAN bus-off conditions. | H | Critical |
|  | Network Recovery | The system shall support recovery following transient network failures. | H | Critical |
|  | Gateway Synchronization | The system shall maintain synchronization with vehicle gateway communication timing. | H | Critical |
|  | Missing Message Detection | The system shall detect timeout of mandatory vehicle communication messages. | H | Critical |
| Manufacturing and End-of-Line Requirements |  |  |  |  |
|  | Manufacturing Test Mode | The system shall support a manufacturing test mode. | M | Non-Critical |
|  | End-of-Line Diagnostics | The system shall support execution of end-of-line diagnostic procedures. | M | Non-Critical |
|  | VIN Storage Support | The system shall support storage and retrieval of vehicle identification information. | M | Non-Critical |
|  | Software Identification Reporting | The system shall provide software identification information. | M | Non-Critical |
|  | Hardware Identification Reporting | The system shall provide hardware identification information. | M | Non-Critical |
| Service Mode Requirements |  |  |  |  |
|  | Service Mode Entry | The system shall support entry into authorized service mode. | M | Non-Critical |
|  | Service Mode Exit | The system shall support exit from service mode. | M | Non-Critical |
|  | Service Function Isolation | The system shall prevent unintended vehicle propulsion during service mode. | H | Critical |
|  | Service Diagnostics Access | The system shall permit diagnostic access while in service mode. | M | Non-Critical |
|  | Service Event Logging | The system shall record service mode events. | M | Non-Critical |
| Connector Requirements |  |  |  |  |
|  | Connector Mating | The system shall support connector mating cycles as specified in the connector supplier specification. | M | Non-Critical |
|  | Connector Lock Detection | The system shall verify connector lock status before enabling operation. | H | Critical |
|  | Connector IP Protection | The system shall use connectors meeting the ingress protection rating specified in the system specification. | H | Critical |
|  | Contact Resistance | The system shall maintain connector contact resistance within the specified limits throughout its service life. | M | Non-Critical |
|  | Connector Presence Detection | The system shall detect connector disconnection during operation within T ms. | H | Critical |
| Compliance to Standard & Regulations |  |  |  |  |
|  | Functional Safety Compliance | The system shall comply with applicable functional safety requirements derived from ISO 26262. | H | Critical |
|  | Cybersecurity Compliance | The system shall comply with cybersecurity requirements derived from ISO 21434. | H | Critical |
|  | AC Charging Compliance | The system shall support conductive charging requirements specified by IEC 61851. | H | Critical |
|  | Communication Compliance | The system shall support EV-EVSE communication requirements specified by ISO 15118. | H | Critical |
|  | EMC Compliance | The system shall comply with applicable automotive EMC requirements. | H | Critical |
|  | Environmental Compliance | The system shall comply with vehicle environmental requirements specified in applicable OEM standards. | H | Critical |
|  | Electrical Safety Compliance | The system shall comply with applicable electrical safety requirements for high-voltage vehicle systems. | H | Critical |
|  | Diagnostic Compliance | The system shall support diagnostics compliant with ISO 14229. | H | Critical |
|  | CAN Compliance | The system shall support communication compliant with ISO 11898. | H | Critical |
|  | Software Update Compliance | The system shall support software update mechanisms compliant with applicable OEM cybersecurity requirements. | M | Non-Critical |
| External Interface Requirement |  |  |  |  |
|  | BMS Interface | The system shall exchange charging-related information with the Battery Management System. | H | Critical |
|  | EVSE Interface | The system shall exchange charging control information with EVSE. | H | Critical |
|  |  | Digital and Analog I/O Functions |  |  |
|  | Diagnostic Interface | The system shall provide diagnostic access through the vehicle diagnostic interface. | H | Critical |
|  | Analog Signal Acquisition | The system shall acquire configured analog signals required for vehicle operation. | H | Critical |
|  | Analog Signal Plausibility Check | The system shall validate analog signal values before use by vehicle functions. | H | Critical |
|  | Digital Input Monitoring | The system shall monitor configured digital inputs. | H | Critical |
|  | Output Command Verification | The system shall verify execution of safety-related output commands when feedback information is available. | H | Critical |
|  |  | Thermal Management Coordination Requirements |  |  |
|  | Cooling Request Management | The system shall request cooling support when aggregate temperatures exceed configured thresholds. | H | Critical |
|  | Cooling Availability Monitoring | The system shall monitor availability of cooling functions. | H | Critical |
|  | Aggregate Thermal Supervision | The system shall supervise thermal status of traction inverters, DCDCs and auxiliary inverters. | H | Critical |
|  | Thermal Derating Activation | The system shall activate power derating when thermal limits are approached. | H | Critical |
|  | Thermal Shutdown Protection | The system shall initiate protective shutdown when thermal limits are exceeded. | H | Critical |
|  | Coolant Information Monitoring | The system shall monitor coolant-related information provided by vehicle systems. | M | Non-Critical |
|  | Aggregate Temperature Reporting | The system shall provide aggregate temperature status to diagnostic systems. | M | Non-Critical |
|  |  | 24V and 12V Battery Management Requirements |  |  |
|  | 24V Battery Monitoring | The system shall monitor status of connected 24V batteries. | H | Critical |
|  | 12V Battery Monitoring | The system shall monitor status of connected 12V batteries. | H | Critical |
|  | Battery Voltage Supervision | The system shall monitor battery voltage throughout vehicle operation. | H | Critical |
|  | Low Voltage Detection | The system shall detect low-voltage battery conditions. | H | Critical |
|  | LV Charging Coordination | The system shall coordinate charging of LV batteries using available DCDC converters. | H | Critical |
|  | Battery Fault Reporting | The system shall report faults affecting LV battery support. | H | Critical |
|  |  | Gateway / HPC Functions |  |  |
|  | HPC Communication Interface | The system shall exchange operational information with the HPC/Gateway controller. | H | Critical |
|  | Vehicle Network Data Exchange | The system shall support exchange of vehicle operational information via CAN and Ethernet networks. | H | Critical |
|  | Network Availability Monitoring | The system shall monitor availability of configured communication networks. | H | Critical |
|  | Communication Fault Propagation | The system shall report network communication failures to vehicle diagnostic systems. | H | Critical |
|  |  | PDU, Pre-Charge and Contactor Requirements |  |  |
|  | Pre-Charge Control | The system shall initiate HV bus pre-charge before energizing HV loads. | H | Critical |
|  | Pre-Charge Verification | The system shall verify successful completion of pre-charge before closing HV main contactors. | H | Critical |
|  | Main Contactor Control | The system shall control HV contactors according to vehicle operating state. | H | Critical |
|  | Contactor Feedback Monitoring | The system shall monitor HV contactor feedback status. | H | Critical |
|  | Contactor Fault Detection | The system shall detect abnormal HV contactor behavior. | H | Critical |
|  | Welded Contactor Detection | The system shall detect indications of a welded contactor condition. | H | Critical |
|  | HV Bus Isolation | The system shall disconnect HV power upon detection of a safety-critical HV fault. | H | Critical |

## Diagnostics Requirements

| SysR ID | Req Desrciption | Priority |
| --- | --- | --- |
| XIN1-SYS-DIA-101 | The Xin1 System shall provide diagnostic communication capability when the vehicle is in a diagnostic-supported operating state. |  |
| XIN1-SYS-DIA-102 | The Xin1 System shall establish diagnostic service availability before accepting diagnostic requests. |  |
| XIN1-SYS-DIA-103 | The Xin1 System shall inhibit diagnostic services that are not permitted in the current operating state. |  |
| XIN1-SYS-DIA-104 | The Xin1 System shall report diagnostic service availability to external diagnostic equipment. |  |
| XIN1-SYS-DIA-201 | The Xin1 System shall detect diagnostic events generated by Xin1 aggregates. |  |
| XIN1-SYS-DIA-202 | The Xin1 System shall classify diagnostic events according to the configured diagnostic strategy. |  |
| XIN1-SYS-DIA-203 | The Xin1 System shall store diagnostic event information for subsequent retrieval. |  |
| XIN1-SYS-DIA-204 | The Xin1 System shall associate diagnostic events with the originating aggregate. |  |
| XIN1-SYS-DIA-205 | The Xin1 System shall maintain diagnostic event records until they are cleared in accordance with the diagnostic strategy. |  |
| XIN1-SYS-DIA-301 | The Xin1 System shall assign a Diagnostic Trouble Code (DTC) to reportable diagnostic events in accordance with the vehicle diagnostic specification. |  |
| XIN1-SYS-DIA-302 | The Xin1 System shall provide stored DTC information to authorized diagnostic equipment. |  |
| XIN1-SYS-DIA-303 | The Xin1 System shall prevent unauthorized modification of stored DTC information. |  |
| XIN1-SYS-DIA-304 | The Xin1 System shall support clearing of DTC information only under conditions defined by the vehicle diagnostic specification. |  |
| XIN1-SYS-DIA-401 | The Xin1 System shall receive supported diagnostic requests from authorized diagnostic equipment. |  |
| XIN1-SYS-DIA-402 | The Xin1 System shall validate diagnostic requests before processing them. |  |
| XIN1-SYS-DIA-403 | The Xin1 System shall reject unsupported or invalid diagnostic requests. |  |
| XIN1-SYS-DIA-404 | The Xin1 System shall transmit diagnostic responses in accordance with the vehicle diagnostic specification. |  |
| XIN1-SYS-DIA-405 | The Xin1 System shall detect communication errors affecting diagnostic services. |  |
| XIN1-SYS-DIA-501 | The Xin1 system shall support UDS diagnostics in accordance with ISO 14229 |  |
| XIN1-SYS-DIA-502 | The Xin1 System shall support diagnostic services defined by the applicable vehicle diagnostic specification. |  |
| XIN1-SYS-DIA-503 | The Xin1 System shall ensure that diagnostic service availability is dependent on the current operating state where required by the vehicle diagnostic specification. |  |
| XIN1-SYS-DIA-504 | The Xin1 System shall ensure that execution of diagnostic services does not compromise vehicle safety. |  |
| XIN1-SYS-DIA-505 | The Xin1 System shall support authenticated diagnostic services where required by the cybersecurity concept. |  |
| XIN1-SYS-DIA-601 | The Xin1 System shall support diagnostic functions required for manufacturing and end-of-line verification. |  |
| XIN1-SYS-DIA-602 | The Xin1 System shall support service diagnostic functions required for maintenance activities. |  |
| XIN1-SYS-DIA-603 | The Xin1 System shall provide diagnostic information necessary to support fault isolation during vehicle service. |  |

## Functional Safety (FuSa) and Cyber Security Requirements

| SysR ID | Req Desrciption | Priority |
| --- | --- | --- |
| Functional Safety Requirements |  |  |
| XIN1-SYS-SAF-101 | The Xin1 System shall monitor the operational status of safety-related aggregates during vehicle operation. |  |
| XIN1-SYS-SAF-102 | The Xin1 System shall detect safety-relevant fault conditions affecting Xin1 aggregates. |  |
| XIN1-SYS-SAF-103 | The Xin1 System shall determine whether detected faults require a safety-related system response. |  |
| XIN1-SYS-SAF-104 | The Xin1 System shall coordinate safety status information across all Xin1 aggregates. |  |
| XIN1-SYS-SAF-105 | The Xin1 System shall transition to a defined safe operating state when required by the applicable safety concept. |  |
| XIN1-SYS-SAF-201 | The Xin1 System shall coordinate safe-state transitions across affected Xin1 aggregates. |  |
| XIN1-SYS-SAF-202 | The Xin1 System shall inhibit vehicle functions that are not permitted in the defined safe operating state. |  |
| XIN1-SYS-SAF-203 | The Xin1 System shall maintain the safe operating state until the defined recovery conditions have been satisfied. |  |
| XIN1-SYS-SAF-204 | The Xin1 System shall classify safety-relevant faults according to the safety concept. |  |
| XIN1-SYS-SAF-301 | The Xin1 System shall coordinate fault responses across affected Xin1 aggregates. |  |
| XIN1-SYS-SAF-302 | The Xin1 System shall prevent conflicting fault responses between Xin1 aggregates. |  |
| XIN1-SYS-SAF-303 | The Xin1 System shall report safety related fault status to external vehicle controllers. |  |
| XIN1-SYS-SAF-304 | The Xin1 System shall coordinate the activation of safety mechanisms required to achieve the defined safe operating state. |  |
| XIN1-SYS-SAF-401 | The Xin1 System shall monitor the availability of safety-related functions required for safe operation. |  |
| XIN1-SYS-SAF-402 | The Xin1 System shall report degradation of safety-related functions to the vehicle. |  |
| XIN1-SYS-SAF-403 | The Xin1 System shall inhibit operation when the minimum required safety functionality cannot be maintained. |  |
| XIN1-SYS-SAF-404 | The Xin1 system shall inhibit propulsion upon detection of traction inverter insulation fault |  |
| XIN1-SYS-SAF-405 | The Xin1 system shall de-rate propulsion upon detection of configurable battery level faults |  |
| XIN1-SYS-SAF-406 | The Xin1 system shall request HV contactor opening for critical battery faults |  |
| XIN1-SYS-SAF-407 | The Xin1 system shall notify the driver of safety-related faults according to vehicle HMI requirements |  |
| XIN1-SYS-SAF-501 | The Xin1 System shall permit recovery from a safety-related fault only when recovery conditions defined by the safety concept are satisfied. |  |
| XIN1-SYS-SAF-502 | The Xin1 System shall verify system readiness before exiting the defined safe operating state following a recoverable fault. |  |
| XIN1-SYS-SAF-503 | The Xin1 System shall report completion of safety-related recovery to external vehicle controllers. |  |
| XIN1-SYS-SAF-601 | The Xin1 System shall detect failures affecting safety-related monitoring functions. |  |
| XIN1-SYS-SAF-602 | The Xin1 System shall record safety-related diagnostic information for service and analysis. |  |
| XIN1-SYS-SAF-603 | The Xin1 System shall make safety-related diagnostic information available through the supported diagnostic interface. |  |
| Cyber Security Requirements |  |  |
| XIN1-SYS-SEC-101 | The Xin1 System shall restrict access to security-sensitive functions to authorized entities. |  |
| XIN1-SYS-SEC-102 | The Xin1 System shall authenticate external entities before permitting execution of protected operations. |  |
| XIN1-SYS-SEC-103 | The Xin1 System shall prevent execution of unauthorized security-sensitive requests. |  |
| XIN1-SYS-SEC-104 | The Xin1 System shall record unauthorized access attempts in accordance with the cybersecurity concept. |  |
| XIN1-SYS-SEC-201 | The Xin1 System shall protect security-relevant communication against unauthorized modification. |  |
| XIN1-SYS-SEC-202 | The Xin1 System shall verify the authenticity of protected communication before use. |  |
| XIN1-SYS-SEC-203 | The Xin1 System shall reject communication that fails authenticity or integrity verification. |  |
| XIN1-SYS-SEC-204 | The Xin1 System shall detect communication anomalies affecting security-relevant assets. |  |
| XIN1-SYS-SEC-301 | The Xin1 System shall protect security-sensitive diagnostic services from unauthorized access. |  |
| XIN1-SYS-SEC-302 | The Xin1 System shall permit execution of protected diagnostic services only after successful authorization. |  |
| XIN1-SYS-SEC-303 | The Xin1 System shall reject unauthorized attempts to modify configuration or calibration data. |  |
| XIN1-SYS-SEC-304 | The Xin1 System shall log security-relevant diagnostic events. |  |
| XIN1-SYS-SEC-401 | The Xin1 System shall verify the integrity of executable software before permitting its execution. |  |
| XIN1-SYS-SEC-402 | The Xin1 System shall prevent execution of software that fails integrity verification. |  |
| XIN1-SYS-SEC-403 | The Xin1 System shall detect unexpected modification of security-relevant software components. |  |
| XIN1-SYS-SEC-501 | The Xin1 System shall accept software update requests only from authorized sources. |  |
| XIN1-SYS-SEC-502 | The Xin1 System shall verify the authenticity and integrity of software update packages before installation. |  |
| XIN1-SYS-SEC-503 | The Xin1 System shall prevent installation of software updates that fail verification. |  |
| XIN1-SYS-SEC-504 | The Xin1 System shall maintain the currently validated software if an update cannot be completed successfully. |  |
| XIN1-SYS-SEC-601 | The Xin1 System shall detect security-relevant events affecting protected assets. |  |
| XIN1-SYS-SEC-602 | The Xin1 System shall classify detected security events according to the cybersecurity concept. |  |
| XIN1-SYS-SEC-603 | The Xin1 System shall record security-relevant events for subsequent analysis. |  |
| XIN1-SYS-SEC-604 | The Xin1 System shall coordinate the configured response to detected security events. |  |
| XIN1-SYS-SEC-605 | The Xin1 System shall report security-relevant events through the supported diagnostic interface where authorized. |  |

## Testing and Validation Requirements

| Req_ID | Req_Name | Req_Description | Priorities (H/M/L) |
| --- | --- | --- | --- |
| MIL Validation Requirements |  |  |  |
| VAL_MIL_001 | MIL Verification Support | The system shall support verification of system functional requirements using a Model-in-the-Loop (MIL) environment. | H |
| VAL_MIL_002 | Functional Behavior Validation | The system shall demonstrate compliance with functional requirements under nominal operating conditions during MIL validation. | H |
| VAL_MIL_003 | Fault Scenario Validation | The system shall support validation of fault handling requirements during MIL simulation. | H |
| VAL_MIL_004 | State Transition Validation | The system shall demonstrate correct execution of defined operating state transitions during MIL validation. | H |
| VAL_MIL_005 | Requirement Coverage | The system shall achieve 100% validation coverage of applicable system requirements allocated to MIL verification activities. | H |
| SIL Validation Requirements |  |  |  |
| VAL_SIL_001 | SIL Verification Support | The system shall support verification of software-executable functions using a Software-in-the-Loop environment. | H |
| VAL_SIL_002 | Functional Consistency Verification | The system shall exhibit behavior equivalent to MIL results for identical test conditions during SIL validation. | H |
| VAL_SIL_003 | Interface Validation | The system shall support validation of software interfaces and communication interactions during SIL testing. | H |
| VAL_SIL_004 | Diagnostic Validation | The system shall support verification of diagnostic functions during SIL validation. | H |
| VAL_SIL_005 | Fault Injection Validation | The system shall support execution of fault injection scenarios during SIL validation. | H |
| VAL_SIL_006 | Regression Verification | The system shall support regression validation following software modifications. | H |
| HIL Validation Requirements |  |  |  |
| VAL_HIL_001 | HIL Validation Support | The system shall support validation using a Hardware-in-the-Loop environment. | H |
| VAL_HIL_002 | Real-Time Execution Verification | The system shall demonstrate compliance with real-time timing requirements during HIL validation. | H |
| VAL_HIL_003 | Communication Verification | The system shall support validation of CAN and Ethernet communication interfaces in HIL environments. | H |
| VAL_HIL_004 | Fault Injection Capability | The system shall support verification of fault response behavior using HIL fault injection methods. | H |
| VAL_HIL_005 | Safe-State Verification | The system shall demonstrate transition to defined safe states under safety-critical fault conditions during HIL validation. | H |
| VAL_HIL_006 | HVIL Validation | The system shall support validation of HVIL fault detection and associated fault reactions during HIL testing. | H |
| VAL_HIL_007 | Propulsion Validation | The system shall support validation of traction inverter functions under simulated operating conditions during HIL testing. | H |
| VAL_HIL_008 | Steering Validation | The system shall support validation of steering inverter related functions during HIL testing. | H |
| VAL_HIL_009 | Brake Pump Validation | The system shall support validation of brake pump inverter related functions during HIL testing. | H |
| VAL_HIL_010 | Charging Validation | The system shall support validation of CCS2 charging communication and charging control functions during HIL testing. | H |
| Lab-car Validation Requirements |  |  |  |
| VAL_LAB_001 | Lab-Car Validation Support | The system shall support integration testing within a Lab-Car environment. | H |
| VAL_LAB_002 | Multi-ECU Integration Validation | The system shall support verification of interactions with external ECUs in a Lab-Car setup. | H |
| VAL_LAB_003 | System Startup Verification | The system shall demonstrate successful startup and shutdown sequences in a Lab-Car environment. | H |
| VAL_LAB_004 | Network Validation | The system shall support validation of network communication under integrated vehicle conditions. | H |
| VAL_LAB_005 | Vehicle State Validation | The system shall demonstrate correct behavior for supported vehicle operating states in Lab-Car testing. | H |
| VAL_LAB_006 | Fault Propagation Validation | The system shall demonstrate proper fault containment and fault propagation behavior during integrated testing. | H |
| VAL_LAB_007 | Redundancy Validation | The system shall support validation of DCDC redundancy and degraded mode operation in Lab-Car testing. | H |
| VAL_LAB_008 | Energy Management Validation | The system shall support verification of load prioritization and power management strategies during Lab-Car validation. | H |
| Vehicle Level Validation Requirements |  |  |  |
| VAL_VEH_001 | Vehicle Validation Support | The system shall support validation in a representative vehicle environment. | H |
| VAL_VEH_002 | End-to-End Functional Validation | The system shall demonstrate compliance with all vehicle-level functional requirements during vehicle testing. | H |
| VAL_VEH_003 | Propulsion Validation | The system shall demonstrate correct operation of dual traction inverter functions during vehicle testing. | H |
| VAL_VEH_004 | Steering Validation | The system shall demonstrate correct operation of steering support functions during vehicle testing. | H |
| VAL_VEH_005 | Brake Validation | The system shall demonstrate correct operation of brake pump support functions during vehicle testing. | H |
| VAL_VEH_006 | Charging Validation | The system shall demonstrate CCS2 charging functionality under vehicle operating conditions. | H |
| VAL_VEH_007 | DCDC Validation | The system shall demonstrate correct operation of 6 kW and 1 kW DCDC functions during vehicle testing. | H |
| VAL_VEH_008 | Gateway Validation | The system shall demonstrate correct CAN and Ethernet gateway functionality during vehicle testing. | H |
| VAL_VEH_009 | Thermal Validation | The system shall demonstrate compliance with thermal performance requirements during vehicle testing. | H |
| VAL_VEH_010 | Functional Safety Validation | The system shall demonstrate compliance with allocated functional safety requirements during vehicle testing. | H |
| VAL_VEH_011 | Cybersecurity Validation | The system shall demonstrate compliance with allocated cybersecurity requirements during vehicle testing. | H |
| VAL_VEH_012 | Crash Event Validation | The system shall demonstrate correct HV shutdown and fault reaction behavior following crash event indications. | H |
| VAL_VEH_013 | Degraded Mode Validation | The system shall demonstrate correct degraded and limp-home behavior during vehicle testing. | H |
| VAL_VEH_014 | Diagnostic Validation | The system shall demonstrate compliance with diagnostic and service requirements during vehicle testing. | H |
| VAL_VEH_015 | Requirement Coverage Validation | The system shall achieve 100% verification status for all allocated vehicle-level requirements prior to release approval. | H |

# User Interface Requirement

| Req ID | Requirement |
| --- | --- |
| XIN1-SYS-UI-001 | System shall provide diagnostic status through service tools. |
| XIN1-SYS-UI-002 | System shall communicate fault information to vehicle cluster. |
| XIN1-SYS-UI-003 | System shall communicate charging status to vehicle displays. |
| XIN1-SYS-UI-004 | System shall provide warning indications for safety-related faults. |

# Considerations/Constraints (Sys / HW)

The following considerations/assumptions/constraints are considered:

- HV operating range shall be compatible with battery architecture.

- Ambient operating temperature shall be -40°C to +85°C.

- Humidity range shall be 10% to 95% RH.

- Communication architecture shall remain CAN compatible.

- Cooling system shall be available during propulsion and charging modes.

- The Battery Management System (BMS) provides validated battery status information.

- The vehicle communication network is available before propulsion enable.

- All external sensors satisfy their respective interface specifications.

- Vehicle actuators comply with their electrical interface requirements.

- The Xin1 System is installed in accordance with the vehicle integration guidelines.

# Other Requirements

## Project Timeline

## Maintenance Schedule

## Calibration & Code Flashing Sequence

The following sequence needs to be followed:

- ECU Identification

- Security Access

- Flash Erase

- Program Download

- Verification

- ECU Reset

## Warranty

Warranty is to be defined by Commercial Vehicle Platform Team.

## Logistical Requirements:

Following are the logical requirements:

- Service tooling support

- Spare part traceability

- Software version traceability

- Manufacturing traceability

# System requirements Signoff details

| Sr.no | Function | Name | Designation |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |

System Requirement Signoff

# Appendix