Software Requirement Document

Project Name: Xin1 Integrated Powertrain Domain Controller

Technology Owner:

Version: 1.0

Document No:  Xin1_SRS_001

Change History

| Version | Date | Author | Reviewer | Approver | Change description |
| --- | --- | --- | --- | --- | --- |
| 001 | 28/07/2026 | Ratneswar Sahoo | Pankaj D | Rajiv Kumar | Initial Draft |

TABLE OF CONTENTS

1	Introduction	4

1.1	Document Scope	4

1.2	Requirements Definition	4

1.3	Terminologies	4

2	Project Requirements	5

2.1	Purpose	5

2.2	Feature List	5

3	System Description	7

3.1	Introduction	7

3.2	Vehicle System Overview	7

3.3	Block Diagram	8

3.4	ECU Interface	8

4	Embedded Software Requirements:	8

4.1	Design Requirements	8

4.2	Diagnostics Requirements	9

4.3	Functional Safety (FuSa) and Cyber Security Requirements	9

4.4	Validation Requirements	9

5	User Interface Requirement	9

6	Considerations/Constraints (SW)	9

7	Other Requirements	9

7.1	Resource Consumption Requirements	9

7.2	Performance Requirements	10

7.3	Operating System Requirements	10

7.4	Project Timeline	10

7.5	Calibration & Code Flashing Sequence	10

7.6	Patches and Updates	11

8	Software Requirements Signoff Details	11

9	Appendix	11

# Introduction

## Document Scope

The purpose of this document is to define the software requirements for the Xin1 Integrated Powertrain Unit Application Software.

The software shall:

- Manage vehicle operational states.

- Coordinate propulsion control.

- Coordinate steering auxiliary inverter operation.

- Coordinate brake compressor inverter operation.

- Supervise DCDC converters.

- Manage charging communication.

- Perform diagnostics.

- Implement functional safety mechanisms.

- Implement cybersecurity functions.

## Requirements Definition

The interpretation of different requirements is controlled by the following definitions:

- Shall, Must - A binding requirement (verifiable) of the component within the scope of this specification.

- Should – A non-binding requirement (recommendation) of the component within the scope of this specification.

- Will - A requirement that is implemented outside of the scope of this specification, that is required for the successful implementation of this specification.

- May – This word in the text expresses a permissible practice or action. It does not express a requirement of the specification

## Terminologies

| Acronym | Definition |
| --- | --- |

Table 1: Terminologies

# Project Requirements

## Purpose

The Xin1 Integrated Powertrain Unit shall integrate propulsion control, auxiliary motor drive control, low-voltage power generation and vehicle control functions within a single enclosure.

The Xin1 system shall:

- Convert HV battery energy into traction power.

- Supply regulated LV power to vehicle loads.

- Control steering pump motor operation.

- Control brake compressor motor operation.

- Coordinate propulsion and regenerative braking.

- Support EV charging communication.

- Ensure functional safety and diagnostic functionality.

- Support commercial vehicle applications including HCV, ILM and Bus variants.

## Feature List

### Stakeholders Description

The following table contains details of all project stakeholders:

| Sr.no | Function | Name | Designation |
| --- | --- | --- | --- |
| 1 | System Engineering |  | System Architect |
| 2 | Functional Safety |  | FuSa Manager |
| 3 | Power Electronics |  | Technical Lead |
| 4 | Software Development |  | SW Architect |
| 5 | Hardware Development |  | HW Architect |
| 6 | Validation |  | Test Manager |
| 7 | Program Management |  | Project Manager |
| 8 | Integration |  |  |

Stakeholder details

### List of features / Broad vehicle level requirements

The given table below contains the list of required features / vehicle level requirements / Applicable PAT requirements.

| Veh_REQ_ID | Vehicle Level Requirement / PAT requirement | Remarks |
| --- | --- | --- |
| Veh_REQ_01 |  |  |

List of Relevant PAT feature List

### Reference documents

#### System Requirement Document

#### System Architecture Design Document

### Reference of standards/regulations

| Document Number | Definition |
| --- | --- |
| ISO 26262 | Functional Safety |
| ISO 21434 | Cybersecurity |
| ASPICE v4.0 | Software Development Process |
| AUTOSAR | Software Architecture |
| ISO 15118 | Charging Communication |
| IEC 61851 | Charging Interface |
| UNECE R155 | Cybersecurity |
| UNECE R156 | Software Update |

List of standards

# System Description

## Introduction

The Software controls and supervises all Xin1 aggregates.

Major software functions include:

- Vehicle State Manager

- Torque Coordinator

- Energy Management

- DCDC Supervisor

- Steering Inverter Control Manager

- Brake Inverter Control Manager

- Charging Manager

- Diagnostics Manager

- Safety Manager

- Cybersecurity Manager

## Vehicle System Overview

The software executes on:

- Main CPU

- Sub CPU

The software communicates with:

- Traction Inverter-1

- Traction Inverter-2

- Steering Inverter

- Brake Inverter

- DCDC #1

- DCDC #2

- DCDC #3

- HVIL

- PLC Interface

- Vehicle CAN Network

## Block Diagram

## ECU Interface

| Interface ID | Description |
| --- | --- |
| SW_IF_01 | CAN Interface |
| SW_IF_02 | Internal CAN |
| SW_IF_03 | PLC Interface |
| SW_IF_04 | HVIL Interface |
| SW_IF_05 | Analog Input Interface |
| SW_IF_06 | Digital Input Interface |
| SW_IF_07 | HSD Interface |
| SW_IF_08 | LSD Interface |

# Embedded Software Requirements:

## Design Requirements

Software_Req_Document

## Diagnostics Requirements

SW_Diagnostics_Req_Doc

## Functional Safety (FuSa) and Cyber Security Requirements

## Validation Requirements

| MIL Validation |  |
| --- | --- |
| SIL Validation |  |
| HIL Validation |  |
| Lab car Validation |  |
| Vehicle Level Validation |  |

# User Interface Requirement

# Considerations/Constraints (SW)

# Other Requirements

## Resource Consumption Requirements

| Memory | Consumption objective |
| --- | --- |

## Performance Requirements

- Efficiency –

- Compliance –

- Coverage –

## Operating System Requirements

### Tasks

| Sr. No. | Tasks | Time (ms) | Description |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |

### Interrupts

| Sr.No. | Interrupt Description | Priority |
| --- | --- | --- |

## Project Timeline

## Calibration & Code Flashing Sequence

| STEP | DESCRIPTION |
| --- | --- |

Calibration and code flashing sequence

## Patches and Updates

# Software Requirements Signoff Details

| Sr.no | Function | Name | Designation |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |

Software Requirement Signoff

# Appendix