Software Requirement Document

Project Name: Xin1 Integrated Powertrain Domain Controller

Technology Owner:

Version: V001

Document No:  Xin1_SRS_001

Change History

| Version | Date | Author | Reviewer | Approver | Change description |
| --- | --- | --- | --- | --- | --- |
| 001 | 28/07/2026 | Ratneswar Sahoo | Pankaj D | Rajiv Kumar | Initial Draft |

TABLE OF CONTENTS

1	Introduction	4

1.1	Document Scope	4

1.2	Requirements Definition	4

1.3	Terminologies	4

2	Project Requirements	5

2.1	Purpose	5

2.2	Feature List	5

3	System Description	7

3.1	Introduction	7

3.2	Vehicle System Overview	7

3.3	Block Diagram	8

3.4	ECU Interface	8

4	Embedded Software Requirements:	8

4.1	Design Requirements	9

4.2	Diagnostics Requirements	9

4.3	Functional Safety (FuSa) and Cyber Security Requirements	9

4.4	Validation Requirements	9

5	User Interface Requirement	10

6	Considerations/Constraints (SW)	10

7	Other Requirements	10

7.1	Resource Consumption Requirements	10

7.2	Performance Requirements	10

7.3	Operating System Requirements	10

7.4	Project Timeline	11

7.5	Calibration & Code Flashing Sequence	11

7.6	Patches and Updates	11

8	Software Requirements Signoff Details	11

9	Appendix	11

# Introduction

## Document Scope

The purpose of this document is to define the software requirements for the Xin1 Integrated Powertrain Unit Application Software.

The software shall:

- Manage vehicle operational states.

- Coordinate propulsion control.

- Coordinate steering auxiliary inverter operation.

- Coordinate brake compressor inverter operation.

- Supervise DCDC converters.

- Manage charging communication.

- Perform diagnostics.

- Implement functional safety mechanisms.

- Implement cybersecurity functions.

## Requirements Definition

The interpretation of different requirements is controlled by the following definitions:

- Shall, Must - A binding requirement (verifiable) of the component within the scope of this specification.

- Should – A non-binding requirement (recommendation) of the component within the scope of this specification.

- Will - A requirement that is implemented outside of the scope of this specification, that is required for the successful implementation of this specification.

- May – This word in the text expresses a permissible practice or action. It does not express a requirement of the specification

## Terminologies

| Acronym | Definition |
| --- | --- |
| EVCU | Electric Vehicle Control Unit |
| BMS | Battery Management System |
| HPC | High Performance Controller |
| CCS2 | Combined Charging System Type-2 |
| HVIL | High Voltage Interlock Loop |
| DCDC | DC/DC Converter |
| OBD | On Board Diagnostics |
| OTA | Over-The-Air Update |
| UDS | Unified Diagnostic Services |
| CAN | Controller Area Network |
| PLC | Power Line Communication |
| PWM | Pulse Width Modulation |
| NVM | Non-Volatile Memory |
| RAM | Random Access Memory |
| MIL | Model in the Loop |
| SIL | Software in the Loop |
| HIL | Hardware in the Loop |
| FuSa | Functional Safety |
| ASIL | Automotive Safety Integrity Level |
| AUTOSAR | AUTomotive Open System ARchitecture |

Table 1: Terminologies

# Project Requirements

## Purpose

The Xin1 Integrated Powertrain Unit shall integrate propulsion control, auxiliary motor drive control, low-voltage power generation and vehicle control functions within a single enclosure.

The Xin1 system shall:

- Convert HV battery energy into traction power.

- Supply regulated LV power to vehicle loads.

- Control steering pump motor operation.

- Control brake compressor motor operation.

- Coordinate propulsion and regenerative braking.

- Support EV charging communication.

- Ensure functional safety and diagnostic functionality.

- Support commercial vehicle applications including HCV, ILM and Bus variants.

## Feature List

### Stakeholders Description

The following table contains details of all project stakeholders:

| Sr.no | Function | Designation |
| --- | --- | --- |
| 1 | System Engineering | System Architect |
| 2 | Functional Safety | FuSa Manager |
| 3 | Power Electronics | Technical Lead |
| 4 | Software Development | SW Architect |
| 5 | Hardware Development | HW Architect |
| 6 | Validation | Test Manager |
| 7 | Program Management | Project Manager |
| 8 | Integration |  |

Stakeholder details

### List of features / Broad vehicle level requirements

The given table below contains the list of required features / vehicle level requirements.

| Veh_REQ_ID | Vehicle Level Requirement |
| --- | --- |
| Veh_REQ_01 | The vehicle shall support electric propulsion using the Xin1 traction power conversion and control functions |
| Veh_REQ_02 | The vehicle shall support controlled regenerative braking through coordination between vehicle control and the traction powertrain |
| Veh_REQ_03 | The vehicle shall receive controlled low-voltage electrical power from the Xin1 DC-DC conversion function for applicable vehicle loads. |
| Veh_REQ_04 | The vehicle shall support electric steering assistance through the Xin1 steering auxiliary inverter and associated motor. |
| Veh_REQ_05 | The vehicle shall support electric brake compressor operation through the Xin1 brake auxiliary inverter and associated motor |
| Veh_REQ_06 | The vehicle shall provide the required vehicle and system wake-up signals to initiate Xin1 operation. |
| Veh_REQ_07 | The vehicle shall provide high-voltage interlock information required for safe high-voltage system operation. |
| Veh_REQ_08 | The vehicle shall support communication between Xin1 and applicable vehicle control systems |
| Veh_REQ_09 | The vehicle shall support EV charging communication through the applicable charging interface |
| Veh_REQ_10 | The vehicle shall detect and manage applicable electrical, thermal and communication faults associated with Xin1 operation |
| Veh_REQ_11 | The vehicle shall provide diagnostic access to applicable Xin1 system information and fault status |
| Veh_REQ_12 | The vehicle shall support controlled transition between defined Xin1 operating states |
| Veh_REQ_13 | The vehicle shall support monitoring of applicable traction, auxiliary and DC-DC system operating parameters |
| Veh_REQ_14 | The vehicle shall support functional-safety-related control and monitoring functions allocated to Xin1. |
| Veh_REQ_15 | The vehicle shall support cybersecurity controls applicable to the Xin1 system and its external interfaces. |
| Veh_REQ_16 | The vehicle shall support calibration and software maintenance activities applicable to Xin1. |
| Veh_REQ_17 | The Xin1 system shall support the applicable vehicle configurations for HCV, ILMCV and Bus applications |

List of Relevant PAT feature List

### Reference documents

#### System Requirement Document

Please refer to “System Requirement Document_SQGP_V001”.

#### System Architecture Design Document

Please refer to “System Architecture Document_SQGP_V001”.

### Reference of standards/regulations

| Document Number | Definition |
| --- | --- |
| ISO 26262 | Functional Safety |
| ISO 21434 | Cybersecurity |
| ASPICE v4.0 | Software Development Process |
| AUTOSAR | Software Architecture |
| ISO 15118 | Charging Communication |
| IEC 61851 | Charging Interface |
| UNECE R155 | Cybersecurity |
| UNECE R156 | Software Update |
| ISO 14229 | Unified Diagnostic Services (UDS) reference |

List of standards

# System Description

## Introduction

The Xin1 Integrated Powertrain Domain Controller consists of a centralized control domain and multiple power conversion domains.

The software is distributed across the Main CPU and Sub CPU according to the system level functional allocation.

The Main CPU provides processing resources for:

- EVCU functions.

- Traction inverter control and coordination.

- Vehicle level coordination functions allocated to the Main CPU.

- Applicable communication, diagnostics, safety and cybersecurity functions.

The Sub CPU provides processing resources for:

- Steering auxiliary inverter control.

- Brake compressor inverter control.

- DCDC converter control.

- Applicable communication, diagnostics, safety and cybersecurity functions allocated to the Sub CPU.

The software interfaces with the power boards, vehicle systems and external interfaces through the hardware and communication interfaces defined by the system architecture.

## Vehicle System Overview

The Xin1 system integrates two major functional domains: Control Domain and Power Domain.

Control domain consists of:

- Main CPU: It performs EVCU and traction control processing functions

- Sub CPU: It performs steering control, brake control and DCDC control processing functions

Power domain consists of:

- Traction Inverter-1

- Traction Inverter-2

- Steering Inverter

- Brake Inverter

- DCDC-1

- DCDC-2

- DCDC-3

The power boards provide the power conversion hardware and associated sensing, gate drive and protection interfaces required by their respective functions.

The control software exchanges commands, feedback and fault information with the applicable power domain elements.

The Xin1 software interacts with external vehicle systems through:

- Digital I/O

- Analog I/O

- HSD/LSD

- HVIL

- CAN

- Ethernet, where applicable

- PLC charging communication.

- PDU / pre-charge interface.

- Cooling system interface.

- Diagnostic/programming interface.

## Block Diagram

## ECU Interface

### External Interface

| Interface ID | Interface | Direction | Description |
| --- | --- | --- | --- |
| SW_ExtIF_01 | CAN | Input/Output | CAN Interface for vehicle network communication |
| SW_ExtIF_02 | HV Battery pack through PDU | Input/Output | HV system status, pre-charge and contactor coordination |
| SW_ExtIF_03 | PLC | Input/Output | Charging communication for CCS2 |
| SW_ExtIF_04 | HVIL | Input | HV interlock status monitoring |
| SW_ExtIF_05 | Analog Input | Input | Acquisition of analog system signals |
| SW_ExtIF_06 | Digital I/O | Input/Output | Vehicle control, wake up, discrete command |
| SW_ExtIF_07 | HSD/ LSD | Input/Output | Control functions for actuators, relays, vehicle loads. |
| SW_ExtIF_08 | Cooling System | Input/Output | Thermal management control |
| SW_ExtIF_09 | Diagnostics | Input/Output | Diagnostics, flashing and calibration |

### Internal Interface

| Source | Destination | Interface purpose | Information exchanged |
| --- | --- | --- | --- |
| Main CPU | Sub CPU | System coordination | System state, commands, status and fault information |
| Main CPU | Traction Inverters | Traction control | Enable, control command, feedback and status |
| Sub CPU | Steering Inverter | Steering control | Enable, control command, feedback and status |
| Sub CPU | Brake Inverter | Brake control | Enable, control command, feedback and status |
| Sub CPU | DCDC Modules | DCDC control | Enable, control command, feedback and status |
| Main CPU | Internal diagnostics/ safety functions | System supervision | Fault, diagnostics and safety status |
| Sub CPU | Internal diagnostics/ safety functions | System supervision | Fault, diagnostics and safety status |

# Embedded Software Requirements:

| Sl.No. | SW Modules/Units |  |
| --- | --- | --- |
|  | Module | Abbreviation |
| 1 | System State & Mode Manager | Xin1_SysState |
| 2 | Vehicle Command Manager | Xin1_VehCom |
| 3 | Torque Control Manager | Xin1_TrqCtrl |
| 4 | Traction Control Manager | Xin1_TcnCtrl |
| 5 | Auxiliary Motor Control Manager | Xin1_AuxCtrl |
| 6 | Steering Control Manager | Xin1_StrCtrl |
| 7 | Brake Control Manager | Xin1_BrkCtrl |
| 8 | DCDC Control Manager | Xin1_DCDCCtrl |
| 9 | Energy / Power Management Manager | Xin1_PowMgmt |
| 10 | Charging Manager | Xin1_ChrgMng |
| 11 | Communication Manager | Xin1_ComMng |
| 12 | Signal & I/O Manager | Xin1_IOMng |
| 13 | Diagnostic Manager | Xin1_DiagMng |
| 14 | Safety Manager | Xin1_SftMng |
| 15 | Cybersecurity Manager | Xin1_CybMng |
| 16 | Fault Manager | Xin1_FltMng |
| 17 | Thermal Manager | Xin1_ThmMng |
| 18 | NVM Manager | Xin1_NvmMng |
| 19 | Software Platform / Service Manager | Xin1_SrvMng |

## Design Requirements

The software requirements defined in this section are derived from the applicable Xin1 System Requirements and allocated to the software architecture elements of the Main CPU and Sub-CPU. 
A single system requirement may result in multiple software requirements where separate software behaviors, conditions, monitoring functions, timing constraints or fault responses are required.

| Sys_Req_ID | Sys_Arch Ele_ID | SW Module | Sw_Req_ID | Req_Desc | Type (Fn/Non-Fn) | Group (Design/Ext/User Int) | Priority | Ctg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| XIN1_SYS_GEN_101 | SYS-CTRL-MAIN | System Initialization & Power Management | XIN1-SW-GEN-101-01 | The software shall support system operation when the supplied input voltage is within the system-defined operating voltage range. | Fn | Design | H | Critical |
| XIN1_SYS_GEN_101 | SYS-CTRL-MAIN | System Initialization & Power Management | XIN1-SW-GEN-101-02 | The software shall monitor the available supply voltage and shall make the voltage status available to affected software functions. | Fn | Design | H | Critical |
| XIN1_SYS_GEN_102 | SYS_CTRL_MAIN | System Initialization & Power Management | XIN1-SW-GEN-102-01 | The software shall detect application of valid vehicle supply power and initiate the system initialization sequence. | Fn | Design | H | Critical |
| XIN1_SYS_GEN_102 | SYS_CTRL_MAIN | System Initialization & Power Management | XIN1_SW_GEN_102_02 | The software shall transition from the power-up condition to the Initialization state within T_PowerUp ms following detection of valid vehicle supply power. | Fn | Design | H | Critical |
| XIN1_SYS_GEN_103 | SYS_CTRL_MAIN | System Initialization & Power Management | XIN1_SW_GEN_103_01 | The software shall execute all software initialization activities required for entry into the operational state. | Fn | Design | H | Critical |
| XIN1_SYS_GEN_103 | SYS_CTRL_MAIN | System Initialization & Power Management | XIN1_SW_GEN_103_02 | The software shall complete the required initialization sequence and permit transition to the operational state within T_StartUp ms after power-up. | Fn | Design | H | Critical |
| XIN1_SYS_GEN_104 | SYS_CTRL_MAIN | Safety & Monitoring | XIN1_SW_GEN_104_01 | The software shall initiate execution of the required start-up diagnostic checks during system initialization. | Fn | Design | H | Critical |
| XIN1_SY_-GEN_104 | SYS_CTRL_MAIN | Safety & Monitoring | XIN1_SW_GEN_104_02 | The software shall evaluate the result of safety-related start-up checks before permitting transition to an operational state. | Fn | Design | H | Critical |
| XIN1_SYS_GEN_105 | SYS_CTRL_MAIN | Vehicle State & Status Management | XIN1_SW_GEN_105_01 | The software shall maintain the current Xin1 system operating state. | Fn | Design | H | Critical |
| XIN1_SYS_GEN_105 | SYS_CTRL_MAIN | Vehicle State & Status Management | XIN1_SW_GEN_105_02 | The software shall provide the current Xin1 system operating state to connected vehicle functions through the applicable communication interface. | Fn | Ext | H | Critical |
| XIN1_SYS_GEN_106 | SYS_CTRL_MAIN | Diagnostics & Event Management | XIN1_SW_GEN_106_01 | The software shall detect reportable system faults and diagnostic events. | Fn | Design | H | Critical |
| XIN1_SYS_GEN_106 | SYS_CTRL_MAIN | Diagnostics & Event Management | XIN1_SW_GEN_106_02 | The software shall request storage of detected reportable fault information in non-volatile memory within T_FltDet s of fault detection. | Fn | Design | H | Critical |
| XIN1_SYS_GEN_107 | SYS_CTRL_MAIN | Diagnostics & Event Management | XIN1_SW_GEN_107_01 | The software shall associate each stored fault event with a valid system timestamp. | Fn | Design | H | Critical |
| XIN1_SYS_GEN_108 | SYS_CTRL_MAIN / SYS_CTRL_SUB | Safety & Monitoring | XIN1_SW_GEN_108_01 | The software shall determine whether a detected fault requires transition to a safe operating state. | Fn | Design | H | Critical |
| XIN1_SYS_GEN_108 | SYS_CTRL_MAIN / SYS_CTRL_SUB | Safety & Monitoring | XIN1_SW_GEN_108_02 | The software shall initiate the applicable safe-state action when a critical fault requiring safe-state entry is detected. | Fn | Design | H | Critical |
| XIN1_SYS_GEN_109 | SYS_CTRL_MAIN / SYS_CTRL_SUB | System Shutdown Management | XIN1_SW_GEN_109_01 | The software shall detect a valid system shutdown request. | Fn | Ext | H | Critical |
| XIN1_SYS_GEN_109 | SYS_CTRL_MAIN / SYS_CTRL_SUB | System Shutdown Management | XIN1_SW_GEN_109_02 | The software shall execute the defined controlled shutdown sequence following receipt of a valid shutdown request. | Fn | Design | H | Critical |
| XIN1_SYS_GEN_110 | SYS_CTRL_MAIN / SYS_CTRL_SUB | System Monitoring | XIN1_SW_GEN_110_01 | The software shall acquire and monitor the system input supply voltage during operation. | Fn | Design | H | Critical |
| XIN1_SYS_GEN_110 | SYS_CTRL_MAIN / SYS_CTRL_SUB | System Monitoring | XIN1_SW_GEN_110_02 | The software shall identify supply-voltage conditions outside the applicable operating limits and make the resulting status available to affected functions. | Fn | Design | H | Critical |
| XIN1_SYS_GEN_111 | SYS_CTRL_MAIN / SYS_CTRL_SUB | System Monitoring | XIN1_SW_GEN_111_01 | The software shall acquire and monitor applicable system temperature information during operation. | Fn | Design | H | Critical |
| XIN1_SYS_GEN_111 | SYS_CTRL_MAIN / SYS_CTRL_SUB | System Monitoring | XIN1_SW_GEN_111_02 | The software shall identify temperature conditions outside applicable operating limits and make the resulting status available to affected functions. | Fn | Design | H | Critical |
| XIN1_SYS_GEN_112 | SYS_CTRL_MAIN | Diagnostics & Event Management | XIN1_SW_GEN_112_01 | The software shall make detected diagnostic events available through the supported diagnostic communication interface. | Fn | Ext | H | Critical |
| XIN1_SYS_GEN_113 | SYS_CTRL_MAIN / SYS_CTRL_SUB | System Monitoring | XIN1_SW_GEN_113_01 | The software shall support monitoring and evaluation of environmental operating conditions applicable to Xin1 operation. | Fn | Design | H | Critical |

Software_Req_Document

## Diagnostics Requirements

| Sys_Req_ID | Sys_Arch_Ele_ID | Req Name | Sw_Req_ID | Flt_Req_Desc | Type(Fn/Non-Fn) | Grp(Design/Ext/User Int) | Priority | Ctg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

SW_Diagnostics_Req_Doc

## Functional Safety (FuSa) and Cyber Security Requirements

## Validation Requirements

| MIL Validation |  |
| --- | --- |
| SIL Validation |  |
| HIL Validation |  |
| Lab car Validation |  |
| Vehicle Level Validation |  |

# User Interface Requirement

# Considerations/Constraints (SW)

# Other Requirements

## Resource Consumption Requirements

| Memory | Consumption objective |
| --- | --- |
| Flash Utilization | ≤ 80% of available memory |
| RAM Utilization | ≤ 80% of available memory |
| CPU Utilization | ≤ 70% average utilization |
| NVM Utilization | ≤ 80% allocation |

## Performance Requirements

- Efficiency –

- Compliance –

- Coverage –

## Operating System Requirements

### Tasks

| Sr. No. | Tasks | Time (ms) | Description |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |

### Interrupts

| Sr.No. | Interrupt Description | Priority |
| --- | --- | --- |

## Project Timeline

## Calibration & Code Flashing Sequence

| STEP | DESCRIPTION |
| --- | --- |

Calibration and code flashing sequence

## Patches and Updates

# Software Requirements Signoff Details

| Sr.no | Function | Name | Designation |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |

Software Requirement Signoff

# Appendix